This information was originally posted by David (“993,951,944” on Rennlist).

I wanted to let everyone know that the 993 idle stabilizer valve is adjustable. When properly adjusted, the results are more bottom end torque and a more stable idle. I only recommend this if your engine is functioning correctly, but the idle is still too low and unstable.

- Remove the idle stabilizer valve. (Clean it as well)

- Remove the epoxy covering the three Allen head screws on the plastic electronic connector side.  I used a sharp very small screw driver to chip away the hard epoxy. Yes, there is an Allen screw (three total) under all that epoxy. You'll need to leave the screws inside as they keep the plastic outer piece in alignment with the inner mechanism.

- Make a scribe marking on the aluminum stabilizer body to the plastic connector side to mark the original factory position.

- Loosen, but do not remove, the three Allen screws. Grip the aluminum body and grab the plastic electrical connector and turn the internal mechanism 2-4 degrees counter-clockwise. (A little less than the distance of one minute on a clocks hand.)Yes, this adjustment is very sensitive! If you can't turn the mechanism, you can use a wrench to increase the grip but be very careful not to damage the connector plastic. Note that by rotating the valve you can see the internal door increase and decrease aperture. Re-install the valve with the three Allen screws slightly loose.

- Start engine and note the idle speed. If it's too high, turn the valve back by turning the electrical connector side (With electrical connector attached) to the factory position, using the scribe marks, and let the engine warm up to normal temperature.  Once the temperature is at normal, slowly turn the valve counter-clockwise until you hear the idle increase slightly. The computer will automatically compensate for the valve position but will allow for some adjustment.  Do not over-adjust as this would result in dangerously high idle and "looping" from computer compensation as well as out of spec emissions.

- Once the optimum valve position is determined, tighten the three Allen screws to hold the valve in the new position. Take a test drive and determine if the idle is normal under all conditions.

Repeat adjustment if needed.
Use the scribe mark to return the valve to the factory position if the results are not helpful.

Some comments from tom_993:

I tried a screwdriver to get the epoxy out, but quickly found that unproductive.  I have a tiny little dentist drill sized grinder attachment for my Dremel tool. This took the epoxy down to the top level of the allen screws in a matter of seconds. The trick is to get the epoxy out of the head of the screw without rounding it out. Since the epoxy is much softer than the steel of the screws, a very gentle touch with the Dremel takes the epoxy away without grinding the screw.

You’ll need a 2.5mm Allen wrench for the screws.

Once I loosened the screws, the mechanism was stuck. I used a large crescent wrench to loosen it. The size and shape spread the force over a large area, and it doesn’t have teeth to mar the plastic. With the wrench, it came loose easily. I made the 2-4 degree adjustment and put it back in the car. I’d say skip this step, as you’ll need to do the adjustment with the car warm and running. With the car running, as you move the connector, the idle will go up and down. I got it as high as 1000 RPM, but finally tightened the screws when I had it set a bit higher than 800. I think the computer does make an adjustment. It seems that turning the car off then back on will cause the computer to re-compute the idle and set the ISV accordingly, as my idle always returned to about 700.