
FUEL - MILEAGE
From: cwhanlon@eai.com 

To: Atifosi944@aol.com 

Subject: Re: ^%^*@$%  Gas Mileage

You might be having a problem with the oxygen sensor.  When the O sensor is not working, the computer will richen up the mixture.  I had an '84 944 that had terrible gas mileage until I replaced the O sensor.  Seems odd that the dealer hadn't suspected that.

----------

The primary mixture signal to the brain is the head temperature sensor.  If it has failed in such a way as to tell the brain that the engine is still cold, when in fact it is warm, you will get lousy mileage, high emissions, and possibly fouled plugs.  It is cheap.

(Dave Cooley)

---------------------------------------------------------------------------------

From: VXLV77B@prodigy.com  (DR JOE R MITCHELL)

Subject: poor mileage

Tonight I've done some sleuthing. A few weeks ago I posted that I'd fixed a poor gas mileage problem that I thought was due to a crankcase vacuum leak. As it turned out I was wrong. I still have the problem. Tonight I checked the DEE system.

The air flow sensor checks out with >8 volts. The air flow sensor door switch however does not. Specified voltage is

    door closed   250‑260 mV        I get 185 mV

    door open      4.6 V            I get 300mV

The other components check out...engine temp switch, and throttle switch. 

So according to Haynes I should chuck the air flow sensor and get a new one. This is also consistent with the manual stating that a DEE system fault can increase gas consumption(running rich due to the fault?). Has anyone else experienced this? Anything else I should look for? Can this be fixed by cleaning the contacts inside the sensor?

I'll call tomorrow for a price on a new sensor but does anyone have a ballpark what this will set me back for just the part as I can install it?

Joe Mitchell, i doc, 84 944

----------

Don`t buy that air flow meter yet!  They are very expensive ($600.00) and rarely fail.  Read the 1st posting for Dec. 20th, `97. (Horsepower tricks).  Let me get the books out and do a little checking, I`ll get back to you, (Tuesday) I think I may have some more info that will help you.   DAVE <superd@direct.ca>

---------------------------------------------------------------------------------

