HEAT SHIELD

Subject: Info on a heat shield, 6/3/99L

From: "Colin Franklin" bfcsm@aol.com 

The San Diego regions newsletter " Windblown Witness" has an article about Porsche's updated heat shield that protects the oil pan gasket from the exhaust manifold. It is about four times the size of the original early turbo shield and does a extraordinary job of protecting the oil pan gasket. It is highly recommended that you install this updated heat shield on all model 944's.  If you would like installation directions let me know.

The part number for the heat shield is 944.100.100.00 the cost is $48.86.

Subject: Re: oil pan gasket heat shield, 9/8/99L

From: "Wesley A. Nicolas" wes@nicolas.org 

The following was written by Steve Grosekemper of the San Diego Region

The redesigned heat shield part number is 944.100.100.00  $46.86

You will also need two 8x12mm bolts and two 8mm lock washers.  

Instructions are to remove the 6mm oil pan bolt at the right rear corner of the oil pan, hold the new heat shield in place and reinstall the 6mm oil pan bolt, finger tight.  Install two 8mm bolts with lock washers through the top mounting holes.  Now tighten all bolts.

Note: Very early 944s will have 10mm threaded mounting holes in the block instead of 8mm holes.  For these cars simply drill out the heat shield mounting holes to accept the 10mm bolts before installation.

The above heat shield is the latest version heat shield (largest) and should provide adequate protection to your oil pan gasket from heat.

