
RACING - RULES
From: "PorscheList@Alpine" <PorscheList@alpinenet.com>

Subject: Club Racing Rules Website

-----------------------------------------------------------------

From: barry.lenoble@peri.com  (Barry Lenoble)

To: bbechtold@smartcamber.com 

Subject: Re: 951 racing

>Could someone who races a 951 in PCA Club racing please help me figure out the >rules.  I think I'd like to run in Improved or Production class with an '86 951.

‑‑‑‑‑‑‑‑‑‑

The rules are very simple. If the rules don't say you can do

something, then you can't. 

There are two classes, stock, and super (AKA GT). 

For the stock classes, you are not allowed very many mods:

1. Engine:

   No mods from air filter to exhaust manifold

   No chip changes

   No changes to injectors or anything like that.

   The AC Compressor must be there, but you can remove the belt

   You can add an oil cooler

   The exhaust system is free (provided you meet noise rules)

2. Suspension: (here it's not so stock)

   Shocks, spring rates, swaybars, are all free

   You can't change the spring type (ie you can't change from torsion bar to coil over)

   Suspension pick up points must remain same

3. Wheels and Tires:

   You can increase wheel size 1" in width. 

   Tires must fit in wheel wells. You can roll the fender lip only

4. Brakes

   Stock calipers and rotors must be used.

   You can groove/slot/drill rotors

   Master clyinder must be stock

   Pad, fluid is free

   You can duct air, and remove backing plates

5. Transmission

   Gear ratios must be stock

   factory LSD allowed

   "Short Throw" OK

   coolers OK

6. Body/Chassis/Interiour

   Chassis/Body, other than bumpers and spoilers, must be the same material and configuration as stock

   Lexan winshield allowed

   Fuel cells allowed

   Control arms free (FABCAR or early steel OK)

   Seats, steering wheel, shift knob free

   Interior must be stock, except for mods for roll bar / cage

That's it for stock cars. If you make certain mods to a stock car, you progress 1 class. For example, running 2" wider wheels, a non stock DME chip, or slotted suspension points would move a 951 from F to E. This is the 'prepared' class.

For the "Super" or "GT" classes, you can pretty much make any change you like. Classes are determined by engine size, with a 1.3 multiplier for turbo cars.

Barry Lenoble, barry.lenoble@peri.com, 89 944 Turbo

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑----------------

